Computer Architecture and Assembly Language Programming (CS401)

Assignment No. 2

BC240432292

AMNA NAWAZ

[ORG 0X0100]

; ========================================================

; Question 1

; Following code closes/terminates the program normally:

; MOV AX, 4C00H

; INT 21H

; Required:

; Change the above code so that the program Terminate and Stay

; Resident (TSR) instead of terminating normally.

; Replace normal termination code with proper DOS TSR service.

; ============================================================

; -------------------Answer 1---------------------------------

MOV DX, (EndProgram + 15) / 16    ; Number of 16-byte paragraphs to keep resident

MOV AX, 3100H      ; AH = 31H DOS TSR service, AL = 00 return code

INT 21H            ; Terminate and Stay Resident

; =============================================================

; Question 2

; Consider the byte-sized array code:

; [ORG 0X0100]

; MOV AL, [Arr]

; MOV [Arr + 4], AL

; MOV AL, [Arr + 2]

; ADD [Arr + 4], AL

; MOV AX, OX4C00

; INT 0X21

; Arr: DB 5, 10, 15, 20, 0

; Required:

; Re-write the above code such that Arr is now defined and

; processed as a word-sized array.

; ==============================================================

; -----------------------Answer 2---------------------------------

MOV AX, [Arr]      ; Load first word value: 5

MOV [Arr + 8], AX   ; Store first word value in 5th word position

MOV AX, [Arr + 4]   ; Load third word value: 15

ADD [Arr + 8], AX   ; Add third word value to 5th word

MOV AX, 4COOH       ; Normal program termination

INT 21H

;--------------------Word-Sized Array----------------------------

Arr: DW 5, 10, 15, 20, 0

EndProgram: